# VF-16 Version 3.10 update

## Major improvements

- Wave file transfer via Plextor CD-R; now you can use the external SCSI CD-R (Plextor PX-W1210TSE is the only compatible SCSI CD-R as of March 2002) to transfer audio to the PC. VF-16 writes wave files on the CD-R, take that CD and put it in CD-ROM player in your PC and import the wave files to your favorite application.
You can also import 44.1Khz/16 bit mono wave files to VF-16.

Now the on-board CD-R has three different functions;

a: FDMS back up; 
To back up audio and mixer (PAN, EQ, EFFECT settings) information to the CD-R, this is proprietary back up, only VF-16 can read the information.

b: Making an audio CD(CD-DA) ; 
Creating audio CDs that can be played back by regular CD players.

c: Export/Import Wave files (44.1Khz/16 bit/ Mono track) ;
In previous version(V.3.04 and before), this function was limited via compatible external SCSI drive , namely Iomega SCSI Zip or SCSI Jaz drive, now this function can be done via the Plextor CD-R.

- Making multiple audio CDs.
You can make more than one audio CDs without re-compiling the list. Please refer to the O/M supplement on details.

- Future software update via Plextor CD-R;
Once the software is updated to V.3.10 and up via compatible SCSI Zip or Jaz drive, the future software update, such as V.3.11, or V.3.12 can be done via Plextor CD-R.

- Various improvement in mixer section;
ADAT Mixer mode, Channel Solo, Fader Pair, Arming 8 tracks at the same time.

- Locate function improvements;
Locates to selected Event Number.
One step locate to the start of the last Record Start point=STOP+REC.

Note: If you have Version V.3.04 and before, you CAN NOT upgrade to V.3.10 via Plextor CD-R, you still have to use SCSI Zip or Jaz drive to update to V.3.10.

If you don’t have SCSI Zip or SCSI Jaz drive, Fostex offers free of charge software update, please send your VF-16 to the following;

Fostex Service Department;

15431 Blackburn Ave, Norwalk, CA90650

Enclose the note that you want to have software update and need an Owner’s Manual Supplement, which explains the new added features. Also your date time phone number. The regular turn around is approximately 2 weeks.